# 1. Read and display a character

.model small

.data

.code

mov ah, 1h      	;   1h is to read a character, character read is stored in al

int 21h  		 ;   this line executes above line

mov dl, al  		;   moving content of al to dl, to print content of dl

mov ah, 2h  		;   to display content of dl

int 21h 		;    to execute above line

end

# 2. Display a string

.model small

.data

str1 db "Hello world$"		; Assigning “ Hello world  $” string to variable str1

.code

main proc

mov bx, @data

mov ds, bx

mov ah, 09h			; To display a string

int 21h

main endp

end

3. Program to read a string and display it.

.model small

.data

str db 100 dup("$")

.code

main proc

mov bx,@data

mov ds,bx

mov dx,offset str

mov ah,0Ah

int 21h

mov dl, 0Ah			; for string to display in next line

mov ah,02h

int 21h

mov dx,offset str

add dx, 02h			; read string stored in str variable character wise

mov ah, 09h			; after reading string character wise display it

int 21h

main endp

end

4. Write a program to add and subtract and display a result.

.model small

.data

.code

main proc

mov dl, 00h

add dl, 30h     ;   converting to ascii number code

add dl, 05h     ;   adding

sub dl, 03h     ;   subtracting

mov ah, 02h     ;   displaying

int 21h

endp

end main

5. To display a numeric number.

.model small

.data

.code

main proc

mov dl, 02h

add dl, 30h     ;to convert to ascii code of a number

mov ah, 02h

int 21h

endp

end main

6. Program to divide a number

.model small

.code

main proc

mov ax, 9h    	;   dividend is 16 bit

mov cl, 2h      ;   divisor is 8 bit

div cl            	;remainder is stored in ah & quotient is stored in al

main endp

end

7.